###### MSTP-REP
Master-Slave/Token-Passing LAN Repeater

## Features and Highlights

- Simple

Just physically connect without any other operation.

- Adaptive

Automatic baud rate recognition from 9600 bps up to 76.8 Kbps.

- Versatile

Designed for BACnet-based MS/TP LANs and other RS-485 based  LANs such as Modbus.

- Flexible

Star-wired bus topology expands MS/TP LAN architecture options.

## Applications and functions

- The Hysine® MSTP-REP is a versatile, high-performance  MS/TP LAN repeater, physically connects two or more BACnet MS/TP LAN segments ,extending your network architecture options while maintaining optimum signal quality and controlling cost.

- The MSTP-REP uses the RS-485 signaling standard over twisted-pair wiring, which can run 1000 meters per segment. So the MSTP-REP gives you flexibility in the design of your BACnet LAN layout. Star-wired topology reduces the installation cost by allowing multiple MS/TP segments to communicate with a single LAN System Integrator.

- The MSTP-REP has terminals for four LAN segments. It can appear anywhere on these segments, with no upstream/downstream requirements.

- The power requirement is broader. With its simple interface and communication rates of up to 76.8 Kbps, MS/TP can be implemented at low cost.

- The MSTP-REP can also be used in other LANs based on RS-485  such as Modbus.

## Ordering information

Item number                              Description                                              .

MSTP-REP                                  Master-Slave/Token-Passing LAN Repeater

## Technical Data

- Power   AC 9~26V @ 8VA or DC 9~32V @ 8VA. Utilizes a half-wave rectifier.

- Communication 4 BACnet MS/TP LAN segments. Switch selectable baud rate from 9.6 to 76.8 Kbps

- Signaling RS–485.

- Dimensions    (120mm)H × (140mm)W × (40mm)D

- Terminations    Removable header-type screw terminals accept 14-24 AWG wire.

- Environmental   -17-70℃.0-95%RH,non-condensing.

- Communications   BACnet MS/TP LAN from 9600 bps up to 76.8Kbps.

- Ratings   EMC GB/T 17626

## Dimension[mm]